| PRODUCT BACKLOG REPORT CeDInt-SEF DataPack – Storey 1 energy-waste finder (TBDA Group 1) Product: a decision tool that turns CeDInt's historical and live sensor data into understandable, located and valued insights on avoidable electricity use: where and when energy is wasted, what it costs in € and CO₂, and what to fix first. This is a Product Backlog in the Scrum sense: an ordered list of stakeholder features, grouped by theme. Items are not assigned to people or planned into future sprints; the Development Team pulls them from the top in each Sprint Planning and breaks them into sprint-backlog tasks (section 6). Status on 19 Oct 2026 · 23 items · 11 High, 7 Medium, 5 Low · 111 story points estimated · 2-week sprints, current Sprint 2 = 12–25 Oct |  | STATUS. Not Started. In Progress. Complete. Overdue. On Hold. |  | PRIORITY. High. Medium. Low. |
| --- | --- | --- | --- | --- |

1. PRODUCT VISION AND SCOPE

Vision: help CeDInt find, value and act on avoidable electricity use, first on historical data and ready to run on the live IoT platform, so the institution can show it has the right tools and capabilities to do the best with its assets.

Why: in 2021–22 about 56 % of Storey 1's non-essential electricity was used while the building was closed (nights, weekends, holidays). A first estimate puts the avoidable part at 5–8 MWh, € 760–1,160 and 0.5–0.8 t CO₂ per year.

Scope of this release: Storey 1 (zones NE, NW, SE, SW), electrical loads by type, hourly data 2020–2023 and 2025, out-of-hours waste and abnormal use, € and CO₂ valuation; final presentation 30 Nov 2026, release on Moodle by 14 Dec 2026.

Out of scope: controlling the building, thermal energy, floors 2–3 (kept as an idea, F6.5).

2. STAKEHOLDERS AND NEEDS

| STAKEHOLDER | NEED | VALUE WE DELIVER (EVIDENCE) |
| --- | --- | --- |
| Building Manager (CeDInt) | Know where and when electricity is used while the building is empty, and what to fix first. | Out-of-hours overview, consumption explorer, floor map, waste alerts ranked by severity. Features F1, F3, F4, F5. |
| Sustainability Officer (UPM) | Credible € and CO₂ figures per zone, month and year for sustainability reporting. | € and CO₂ per alert, monthly and yearly savings report with export. Feature F5. |
| Maintenance Technician | Tell real equipment problems apart from faulty meters, and know where and when to act. | Separate data-error alerts, abnormal-use alerts during the day, alert handling. Features F2, F4. |
| UPM / CeDInt Sponsor (Product Owner) | Evidence that the product works, an auditable method, and a basis to decide on deployment. | Business case, justified model choice, deployment plan, demo mode, demo video; quality requirements Q1 and Q8. Features F3, F6. |

3. LOADS IN SCOPE: DECISION, JUSTIFICATION AND PRIORITY

The brief leaves open which loads to analyse. We decided from the Sprint 1 profiling of the 18 series (about 1.34 million rows) and the first waste estimate, prioritising by value for the Building Manager and by data quality.

| LOAD | DECISION | JUSTIFICATION (FROM THE DATA) | PRIORITY | WHAT THE PRODUCT OFFERS |
| --- | --- | --- | --- | --- |
| Plugs & Sockets (all 4 zones) | In scope – core | 72 % of the energy and about 70 % of the estimated waste (4.7 MWh/yr). NW plug load at night is 2.8× higher in winter than in summer: probably heaters left on in empty rooms. | High | Waste alerts, € and CO₂, expected consumption |
| Lighting (zones NW, SE, SW) | In scope – core | 15 % of the energy, 1.4 MWh/yr estimated waste. NW lighting follows a timer pattern every day, incl. weekends: possibly security lighting, to be confirmed (section 7). | High | Waste alerts, € and CO₂ |
| HVAC (zones NE, NW, SE) | In scope | 5.5 % of the energy, 0.4 MWh/yr estimated waste; the most weather-dependent load, so it drives the expected-consumption model. | Medium | Expected consumption, abnormal-use alerts |
| Hand dryers (NW, SW) | In scope – secondary | Small, event-driven load (0.04 MWh/yr waste). | Low | Shown, rarely alerts |
| Emergency lighting, RACK | Excluded from waste | Must run 24/7 (safety, servers). Monitored only for data errors: NW emergency lighting is negative 99 % of the time (probably a reversed meter); SW RACK stops in Dec 2021. | n/a | Data-error alerts only |
| SE Projector | Out of scope | Series is 100 % zeros: no information. | n/a | Reported as a data issue |

4. PRODUCT BACKLOG

Items are grouped by feature (theme) for readability. The order of work is the rank under each ID (1 = next to be pulled), set by the Product Owner by business value. SPRINT is filled in only for items the Development Team has already selected in a Sprint Planning; "–" means not selected yet. ESTIMATE is the Development Team's own estimate in story points (sum of its task estimates); new items are estimated during refinement.

| ID | AS A … | I WANT TO … | SO THAT … | PRIORITY | ESTIMATE (SP) | SPRINT | STATUS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F1. SEE WHERE AND WHEN ENERGY IS USED |  |  |  |  |  |  |  |
| F1.1 rank 1 | Building manager | see how much electricity each zone and load type uses while the building is closed (nights, weekends, holidays) | I know where the largest saving opportunities are | High | 10 | 2 | In Progress |
| F1.2 rank 3 | Building manager | browse hourly, daily and monthly consumption per zone and load type with shared zone and date filters | I can check any pattern myself | High | 11 | 2 | In Progress |
| F1.3 rank 16 | Building manager | see a floor map of Storey 1 with the four zones coloured by waste in the chosen period | I see at once where to go | Medium | 3 | – | Not Started |
| F1.4 rank 19 | Building manager | see a day × hour heatmap per zone and for the whole floor | I spot recurring patterns such as lights left on every Friday night | Low | 2 | – | Not Started |
| F2. TRUST THE DATA |  |  |  |  |  |  |  |
| F2.1 rank 2 | Building manager | see how complete and reliable the data are for every zone, load type and month | I never act on a number caused by missing data or a meter reset | High | 7 | 2 | In Progress |
| F2.2 rank 10 | Maintenance technician | see data errors (dead meters, flat lines, counter resets) as a separate kind of alert | I can repair faulty meters and they are not reported as waste | High | 2 | – | Not Started |
| F3. KNOW WHAT CONSUMPTION IS NORMAL |  |  |  |  |  |  |  |
| F3.1 rank 4 | Building manager | see the expected consumption of each zone and load type for every hour, next to the actual consumption | I recognise unusual use at a glance | High | 6 | 2 | Not Started |
| F3.2 rank 9 | Building manager | have the expected consumption take outdoor weather, holidays and the academic calendar into account | cold days or exam periods do not cause false alarms | High | 12 | 2 | Not Started |
| F3.3 rank 13 | UPM / CeDInt sponsor | see the chosen forecasting model compared with alternatives and documented in a model card | I can trust that the best available model is used | Medium | 5 | – | Not Started |
| F4. DETECT WASTE AND ANOMALIES |  |  |  |  |  |  |  |
| F4.1 rank 5 | Building manager | get an alert whenever a zone uses more than its standby level while the building is closed | I can have left-on equipment switched off | High | 12 | 2 | In Progress |
| F4.2 rank 11 | Building manager | open a list of all alerts and the detail of each one | I understand what happened before I act | High | 7 | – | Not Started |
| F4.3 rank 12 | Maintenance technician | get an alert when consumption during opening hours is far above what is expected | I also find faulty or misused equipment during the day | Medium | 8 | – | Not Started |
| F4.4 rank 15 | Maintenance technician | mark an alert as handled, false alarm or data error | the team sees what was done and false alarms improve the thresholds | Medium | 4 | – | Not Started |
| F4.5 rank 21 | Building manager | receive a weekly e-mail with the most severe alerts | I do not have to open the dashboard every day | Low | – | – | Not Started |
| F5. VALUE AND PRIORITISE |  |  |  |  |  |  |  |
| F5.1 rank 6 | Sustainability officer | see the cost in € and the kg CO₂ of every alert, with totals per day, month and zone | I can rank problems by impact and report them | High | 4 | 2 | In Progress |
| F5.2 rank 7 | Building manager | see alerts ranked as Low, Medium or High | I know what to handle first | High | 2 | 2 | In Progress |
| F5.3 rank 14 | Sustainability officer | get a monthly and yearly savings report per zone and load type, with the top waste sources, that I can export | I can include it in sustainability reporting | Medium | 5 | – | Not Started |
| F6. DECIDE, DEPLOY AND SHARE |  |  |  |  |  |  |  |
| F6.1 rank 8 | UPM / CeDInt sponsor | receive a business case with problem, solution, savings potential, costs and benefits | I can decide whether to deploy the product | High | 6 | 2 | In Progress |
| F6.2 rank 17 | UPM / CeDInt sponsor | get a plan for running the product on CeDInt's live IoT data | I know what deployment would take | Medium | 2 | 2 | In Progress |
| F6.3 rank 18 | UPM / CeDInt sponsor | run the product in a demo mode that replays historical data as if it were live | it can be shown without a live data connection | Medium | 2 | – | Not Started |
| F6.4 rank 20 | UPM / CeDInt sponsor | have a short demo video of the product | the message "our organisation has the right tools to do the best with your asset" reaches prospective clients | Low | 1 | – | Not Started |
| F6.5 rank 22 | Building manager | get the same analysis for floors 2 and 3 | the whole building is covered | Low | – | – | On Hold |
| F6.6 rank 23 | Building manager | have alerts produced automatically from the live IoT platform | problems are found the same day | Low | – | – | On Hold |